## How to use Basic Control

BasicControl is a single jar file that contains classes to control a Pioneer robot and to get data from an (LMS100) laser scanner. It runs a loop which looks for external controlling methods (e.g. a navigation method). A joystick controller is automatically added with a switchable priority between 0 and 100, hence an external controller can be added with e.g. priority 50 which then can be triggered by the joystick.

A program that wants to use BasicControl needs to

- add the BasicRobotControl.jar file to the project

- add  the

- import the basicrobotcontrol package

To connect to robot and laser, instantiate a MainControl object, calling the constructor with a parameter for the com-port to the Pioneer robot

- MainControl mc = new MainControl(args); // e.g. args = {{"COM6"}} .

Note about com ports: in windows, only COM0-COM9 can be used. This seems to be a bug in ARIA. I don't know about other operating systems.

## How to add your own (external) controller:

The instantiated MainControl object mc has a method "addControl(RobotController rc)". Call this method, e.g. using

MainControl mc = new MainControl(args);

MyController myc = new MyController();

mc.addController(myc);

to register your controller to the main controller class.  In order to use the addController method, the external controller class must implement the RobotController interface. It declares two methods:

package basicrobotcontrol;

public interface RobotController {

public double[] getMotorSpeedLR();

public int getControlPriority();

}

GetMotorSpeed returns a double[2] array for left and right motor speed. This value is sent to the motor by the mc object. Please make sure that access to the array is threadsafe, if your controller runs its own thread. Here is an example implementation:

// RobotController (interface) methods

public double[] getMotorSpeedLR() {

double[] cslr = new double[2];

lock.lock();	// lock is a ReentrantLock

cslr[0] = speedLR[0];

cslr[1] = speedLR[1];

lock.unlock();

return (cslr);

}

public int getControlPriority() {

return (priority);

}

The values which are returned are only sent to the mopors if the priority of the controller is the highest of all controllers registered.

## Getting Odometry

The robot is accessible through the field "public PioneerRobot robot" in MainControl .  PioneerRobot objects own the method

public double[] getRelativePose()

It returns the pose in mm and degrees, the coordinate system is: X=>forward, Y: left

## Getting Laser Data

The laser is accessible through the field " public LMS100 laser" in MainControl. Currently, the laser is automatically started in a Thread, repeatedly querying the laser. The data from this thread can be obtained using

public double[][] getLastDataXY(boolean readNew)

setting readNew to false. Setting readNew to true obtains a new laser reading. If your application is not overly time critical, the Thread-provided readings should be used to avoid multiple readings from the laser through different controllers.

getLastDataXY is threadsafe, the returned data is a deep copy of the reading.

## Displaying Laser related data

The Laser thread creates a JPanel, which is displayed by the MainControl class. In order to draw data into this panel, classes can register to the LMS100 class as "LaserPainter" objects. They have to implement the LaserPainter interface, and must register to the LMS100 using the method "addLaserPainter(LaserPainter lp)" in LMS100. As an example, see the LineCreator class.