Assignment 3 by Tatyana Nuzhnaya

- Show that (A c B)' = (A' o B), (A o B)' = (A' c B); c – closing, o – opening, + - dilation, - - erosion

In class we showed A + B = ( A'- B)' (*)

1)
(A c B)' = ((A + B) - B)' 
(A' o B) = (A' - B) + B = ((A' - B) ' - B)' = ((A + B) - B)'

Therefore (A c B)' = (A' o B) 

2)
(A o B)' = ((A - B) + B)' =(A-B)' -B
(A' c B) = (A' + B) - B = (A-B)' - B

Therefore (A o B)' = (A' c B)

The original Fingerprint image:

To determine bifurcations on the original image we perform the following steps:

- Thinning fingerprint image untill stability(there are no more changes, the dges are 1 pixel wide)

- Determine bifurcation points using hit and miss transformations and predefined branching patterns.

There are 6 possible branching patterns(0 – black pixel, 1- white pixel)

| 0,1,0 1,0,1 0,1,0 | 0,1,0 1,0,1 1,1,0 | 1,0,1 0,0,1 1,1,0 |
| --- | --- | --- |
| 1,1,0 0,0,1 1,1,0 | 1,1,0 0,0,0 1,1,0 | 0,0,0 1,0,1 0,1,1 |

All branching patterns can be rotated 90 degrees up to 4 times.

The final determined bifurcations are:

The code:

I=imread('fingerprint.jpg');

J=I(:,:,1)>160;

K=bwmorph(~J,'thin','inf');

skeleton=~K;

imshow(skeleton);

pattern1=[0,1,0;1,0,1;0,1,0];% cross type branching

pattern2=[1,1,0;0,0,1;1,1,0];% almost t-type branching

pattern3=[0,1,0;1,0,1;1,1,0];% half cross

pattern4=[1,1,0;0,0,0;1,1,0];% t-type

pattern5=[1,0,1;0,0,1;1,1,0];%

pattern6=[0,0,0;1,0,1;0,1,1];%

branching_point = bwhitmiss(skeleton,pattern1,~pattern1);

[row,col]=find(branching_point==1);

%figure;imshow(I);

for i=1:size(row,1)

hold on;

plot(col(i),row(i),'r+');

end

for i=1:4

branching_point = bwhitmiss(skeleton,pattern2,~pattern2);

[row,col]=find(branching_point==1);

for j=1:size(row,1)

hold on;

plot(col(j),row(j),'ro')

end

pattern2=rot90(pattern2);

end

for i=1:4

branching_point = bwhitmiss(skeleton,pattern4,~pattern4);

[row,col]=find(branching_point==1);

for j=1:size(row,1)

hold on;

plot(col(j),row(j),'ro')

end

pattern4=rot90(pattern4);

end

for i=1:4

branching_point = bwhitmiss(skeleton,pattern5,~pattern5);

[row,col]=find(branching_point==1);

for j=1:size(row,1)

hold on;

plot(col(j),row(j),'ro')

end

pattern5=rot90(pattern5);

end

for i=1:4

branching_point = bwhitmiss(skeleton,pattern3,~pattern3);

[row,col]=find(branching_point==1);

for j=1:size(row,1)

hold on;

plot(col(j),row(j),'ro')

end

pattern3=rot90(pattern3);

end

for i=1:4

branching_point = bwhitmiss(skeleton,pattern6,~pattern6);

[row,col]=find(branching_point==1);

for j=1:size(row,1)

hold on;

plot(col(j),row(j),'ro')

end

pattern6=rot90(pattern6);

end