Find the Gear Using Fourier Analysis

Tianyang Ma

The code entrance is gear.m

- With morphology operation, the gap of original image will be filled.

- Using morphology bwmorph command, extract the boundary, and use bwtraceboundary to get the order of the edge.

- Using CentroidDistance to compute the distance from each edge point to centroid point.

- With FFT analysis, we decompose the distance signal into several frequencies, and compute the spectrum.

- Find the spectrum peak except the constant signal, if there is another peak, it’s a gear.

Several result images:

There are totally 14 shapes in the test image, all of their distance time series curve and spectrum is plotted out, and saved in the folder.

However, Gear.m  would only display the two gears shape, and their spectrum.